Screen Capture by PC Tool

1. Set “Logo” to “User”

Set the “Logo” to “User” in Setup options menu.

2. Enter download mode

Hold both "Power" and "Menu" buttons and then plug in power cord. Release the buttons until Power/Lamp/Temp LED indicators are lighting red.

3. Connect USB cable between PC and projector

Please find the “SERVICE” USB port and plug-in USB cable.

4. Click Splash Downloader

Click Splash Downloader. If the message says “Projector found”, the projector has been connected successfully.

Note: If “Error: No Projector found.”, the projector is not connected. Please retry step 2 and step 3.

5. Select logo file

Click the “Browse” button and select logo file. The recommended resolution of logo should not exceed the default logo resolution (For example, logo resolution is 1920x1200). The purpose is that keeping the quality of the image. If not, the logo may be blurred.

Note: Support “PNG” & “BMP” file format.

6. Download logo to projector

Click the “Flash Image” button. If the file was written to projector successfully, the message “Download Complete” will be displayed on the screen.

7. Reboot projector

Plug-out the power cord of projector and plug-in it. Please ensure that the logo has been upgraded correctly.

8. Operating system compatibility